Viscosities of binary mixtures containing 1-butanol + 2,2,4-trimethylpentane or + 1,2,4-trimethylbenzene at high pressures for the thermophysical characterization of biofuels

Johnny Zambranoa,b, M. Carmen Martína, Ángel Martínc, José J. Segoviaa,*

a TERMOCAL (Thermodynamics and Calibration) Research Group, University of Valladolid. Escuela de Ingenierías Industriales, Paseo del Cauce 59, 47011 Valladolid, Spain.

b Escuela Politécnica Nacional, Fac. de Ing. en Geología y Petróleos, Dep. de Petróleos, Quito, Ecuador.

c High Pressure Process Group, Department of Chemical Engineering and Environmental Technology, University of Valladolid, Valladolid, Spain.

* To whom correspondence should be addressed. E-mail: jose.segovia@eii.uva.es

Tel./Fax: +34 983184690

ABSTRACT

A wide variety of oxygenated additives from renewable origin may be used in next-generation biofuels.

In order to contribute to the international effort aimed at developing and using environmentally sustainable fuels, the present study focuses on the thermophysical characterization of new blends of hydrocarbons and 1-butanol (known as a second generation biofuel) through viscosity measurements.

A vibrating-wire viscometer was developed to accurately measure dynamic viscosities at T = (293.15 to 373.15) K and p = (0.1 to 140) MPa. The relative expanded uncertainty of the dynamic viscosity was estimated at 1.5% for a coverage factor k = 2.

As an initial characterization of biofuels, the dynamic viscosities of (1-butanol + 2,2,4-trimethylpentane) and (1-butanol + 1,2,4-trimethylbenzene) mixtures, as well as 1-butanol, are reported at different temperature and pressure conditions.

Keywords: 1-butanol; 2,2,4-trimethylpentane; 1,2,4-trimethylbenzene; viscosity; high pressure.

1.	Introduction

According to an official communication from the European Union (EU) in 2006 [1], estimated emissions from road transport accounted for 21% of all greenhouse gas emissions. Since 2006, European energy policy has examined Europe's overdependence on imported oil and gas and has developed a consistent approach aimed at gradually  reducing this dependency.

The EU established a strategy for biofuels wherein one of the main objectives was to research into “second generation” biofuels in an effort to support their introduction into the market by increasing demonstration projects and removing non-technical obstacles.

Second generation biofuels are produced from non-food biomass, such as ligno-cellulosic materials which include by-products, wastes and dedicated feedstocks [2], and are an effective alternative to fossil fuels in addition to offering advantages over first generation fuels [3].

According to Szwaja and Naber [4], butanol can replace gasoline as a pure fuel or when blended with gasoline in internal combustion engines since its properties resemble those of gasoline. There are models covering a wide range of compositions, temperatures and pressures, which are capable of predicting properties such as phase equilibrium, enthalpy, heat capacity, etc. However, there remains a great need for reliable predictions and accurate thermodynamic as well as thermophysical behaviour data [5].

Our research group is engaged in this task and some measurements of mixtures of alcohol + hydrocarbon have been performed such as vapour-liquid equilibria [6,7], excess enthalpies [8], densities and heat capacities [9]. In this paper, dynamic viscosities are reported for the binary mixtures (1-butanol + 2,2,4-trimethylpentane) and (1-butanol + 1,2,4-trimethylbenzene) at four temperatures (293.15, 313.15, 333.15, 353.15) K, pressures up to 140 MPa and mole fractions of 1-butanol x = (0.3, 0.5, 0.8, 1). Viscosity data of the pure hydrocarbons have already been published [10].

2.	Experimental

2.1.	Materials

1-Butanol and 2,2,4-trimethylpentane were purchased from Sigma-Aldrich, with a purity >0.995 by gas chromatography (GC). 1,2,4-Trimethylbenzene was purchased from Aldrich Chemistry, with a purity >0.98. They were used as provided and checked by gas chromatography. Their characteristics are summarized in Table 1.

Table 1. Material description.

| Chemical name | Source | Mass fraction Purity (GC)a | Purification method |
| --- | --- | --- | --- |
| 1-butanol | Sigma-Aldrich | 0.995 | None |
| 2,2,4-trimethylpentane | Sigma-Aldrich | 0.995 | None |
| 1,2,4-trimethylbenzene | Aldrich Chemistry | 0.98 | None |

a From the supplier

2.2 Apparatus and procedure

A vibrating-wire viscometer [10,11], capable of working at T = (288.15 to 423.15) K and pressures up to 140 MPa, was developed to accurately measure dynamic viscosities up to 35 mPa·s. The vibrating wire and magnetic assembly are placed in a pressure vessel immersed in a Hart Scientific 6020 thermostatic bath. Fluid temperature is measured using two platinum resistance thermometers (PRT) and a ASL F100 thermometer calibrated with a standard uncertainty of ± 0.01 K. A Druck DPI 104 transducer allows pressure measurement to a standard uncertainty of ± 0.01%. Both devices were calibrated in our laboratory using primary standards and measurements are traceable to national standards.

The technique uses tungsten wire (length 50 mm and nominal radius 75 µm) anchored at both ends [12,13] as a sensor and placed inside a pressure vessel, with an external constant magnetic field. A detailed description of this technique, its set-up and calibration as well as uncertainty calculation is given in a previous paper [10]. The relative expanded uncertainty of the viscosity measurements is estimated better than ± 1.5% for a cover factor k = 2.

The measurement principle and working equations of vibrating-wire viscometers are well known and described in the literature [12-15]. The technique was calibrated with toluene and checked with dodecane. Heptane and other hydrocarbons such as 2,2,4-trimethylpentane and 1,2,4-trimethylbenzene were also measured [10].

Viscosity data at atmospheric pressure are usually correlated using the Vogel [16], Fulcher [17] and Tammann [18] equation (VFT), obtained by performing some assumptions [19,20] from Adam-Gibbs’ theory of configurational entropy [21]:

ηT=A·expBT-C=A·expD·ToT-To						(1)

D is the index of frailty (or Angell’s parameter) and To Vogel’s temperature, where the parameters C = To and B = DTo are independent of pressure. This equation was extended to account for the effect of pressure by Comuñas et al. [22,23], given by equation (2), and was used in this paper:

η(p,T)=A·expBT-C·p+E(T)pref+E(T)F						(2)

parameters A, B and C are determined through a preliminary viscosity setting as a function of temperature at the reference pressure (pref = 0.1 MPa, in this study); E(T)=E0+E1T+E2T2 and the coefficients F, E0, E1 and E2 are obtained from the viscosities at pressures different from the reference one.

3.	Results and discussion

As an initial thermophysical characterization of biofuels, dynamic viscosity of 1-butanol and its mixture with different hydrocarbons (2,2,4-trimethylpentane or 1,2,4-trimethylbenzene) are reported at different temperatures, pressures and mole fractions of 1-butanol x = (0.3, 0.5, 0.8, 1.0).

Dynamic viscosity measurements were carried out at four isotherms in the range T = (293.15 – 353.15) K and pressures up to 140 MPa. Additionally, viscosity measurements were performed using a Stabinger Viscometer at 0.1 MPa for 1-butanol in order to check the vibrating wire viscometer. Table 2 shows the experimental viscosities of 1-butanol and Figure 1 shows the dynamic viscosity of the 1-butanol at different temperatures and pressures.

As is known, viscosity decreases with temperature and increases with pressure. For 1-butanol, a decrease of 73% is observed when temperature rises from 293.15 K to 353.15 K for all the measured pressures. However, increases in viscosity due to pressure, when this rises from 0.1  to 140 MPa, ranges from 149% at T = 333.15 K to 164% at  T = 293.15 K.

Table 2: Dynamic viscosities of 1-butanol,  (mPa·s), at different temperatures and pressures.a

|  | / mPa·s |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| p /MPa |  |  |  | T /K |  |  |  |
|  | 293.15 | 298.15 | 313.15 | 323.15 | 333.15 | 348.15 | 353.15 |
| 0.1b | 2.9590 |  | 1.7940 |  | 1.1543 |  | 0.7837 |
| 0.1 | 2.9686 | 2.6013 | 1.7968 | 1.4288 | 1.1564 | 0.8579 | 0.7903 |
| 1.0 | 3.0011 | 2.6221 | 1.8132 | 1.4401 | 1.1722 | 0.8647 | 0.7943 |
| 5.0 | 3.1084 | 2.7151 | 1.8899 | 1.4910 | 1.2273 | 0.8956 | 0.8284 |
| 10.0 | 3.2435 | 2.8337 | 1.9672 | 1.5559 | 1.2905 | 0.9349 | 0.8637 |
| 20.0 | 3.4787 | 3.0785 | 2.1399 | 1.6899 | 1.3888 | 1.0161 | 0.9473 |
| 30.0 | 3.7881 | 3.3335 | 2.3189 | 1.8294 | 1.5041 | 1.1007 | 1.0405 |
| 40.0 | 4.0777 | 3.5987 | 2.4799 | 1.9745 | 1.6240 | 1.1887 | 1.1017 |
| 50.0 | 4.4007 | 3.8741 | 2.6295 | 2.1251 | 1.7414 | 1.2801 | 1.2100 |
| 60.0 | 4.7398 | 4.1596 | 2.8520 | 2.2813 | 1.8671 | 1.3749 | 1.2952 |
| 70.0 | 5.0764 | 4.4554 | 3.0886 | 2.4431 | 2.0137 | 1.4730 | 1.3838 |
| 80.0 | 5.4377 | 4.7613 | 3.2466 | 2.6104 | 2.1367 | 1.5746 | 1.4734 |
| 100.0 | 6.1645 | 5.4037 | 3.6858 | 2.9616 | 2.3839 | 1.7879 | 1.6762 |
| 120.0 | 6.9795 | 6.0869 | 4.1920 | 3.3351 | 2.6327 | 2.0149 | 1.8566 |
| 140.0 | 7.8435 | 6.8109 | 4.6383 | 3.6822 | 2.8831 | 2.2554 | 2.0372 |

a Standard uncertainties (k=1): ur(p) = 0.0002 kPa/kPa; u(T) = 10 mK; ur() = 0.0075 mPa·s/mPa·s.

b Measured using Stabinger viscometer. Standard uncertainties (k=1): ur(p) = 0.005 kPa/kPa; u(T) = 10 mK; ur() = 0.005 mPa·s/mPa·s.

Figure 1. Dynamic viscosities of 1-butanol as a function of pressure at different temperatures: ◊ 293.15 K; □ 298.15 K;  313.15 K;   323.15 K; * 333.15 K; ○ 348.15 K;  353.15 K. Symbols represent experimental data and solid lines represent VFT correlation (equation (2)).

Experimental data were correlated using the modified VFT model, equation (2), as explained above, obtaining an average absolute deviation of 1.1%. Relative deviations, obtained with the correlation in comparison with experimental values, are depicted in Figure 2 as a function of pressure at different temperatures. The fitting parameters and standard deviation are summarized in table 3.

Table 3: Correlation parameters of modified VFT model, equation (2), and standard deviation () for 1-butanol.

| A/ | B/ | C/ | E0/ | E1/ | E2/ | F | ()/ |
| --- | --- | --- | --- | --- | --- | --- | --- |
| mPa·s | K | K | MPa | MPa·K-1 | MPa·K-2 |  | mPa·s |
| 0.011849 | 1068.232 | 99.922 | 0.946033 | 0.961762 | -0.00126 | 1.633896 | 0.037 |

Figure 2. Relative deviations of viscosities in comparison with the values calculated using modified VFT equation (eq. 2) as a function of pressure at different temperatures: ◊ 293.15 K; □ 298.15 K;  313.15 K;   323.15 K; * 333.15 K; ○ 348.15 K;  353.15 K.

Experimental viscosities measured using the vibrating wire viscometer were compared at 0.1 MPa with those obtained using a Stabinger viscometer, with an average absolute relative deviation of 0.2% being obtained. We also compared our measurements using the vibrating wire viscometer with the values found in the literature [9, 24-33], all of them being measured at 0.1 MPa. Relative deviations range from -0.06% [26] to +2.3% [31], as can be seen in figure 3, where relative deviations are plotted as a function of the experimental viscosity. The absolute average relative deviations obtained were: 1.3% [9]; 1.2 % [24]; 0.2 % [25]; 0.6% [26]; 1.4 % [27]; 0.1 % [28]; 1.0 % [29]; 0.2 % [30]; 1.0 % [31]; 1.0 % [32] and 0.8 % [33]. There are four points for comparison with the data published by Zivkovi et al. [31] and Knezevic-Stevanovic et al. [33].  These results confirm the agreement with our data within the measurement uncertainties.

Figure 3. Relative deviations of viscosities in comparison with literature values as a function of the experimental viscosity measured in this work: ◊ measured in this work using a Stabinger-viscometer; □ Torín-Ollarves et al. [9]; ▲ Riddick et al. [24]; ■ Saleh et al. [25]; ● Wankhede et al [26];  Chen et al. [27];  Rostami et al. [28];  Shadegui et al. [29]; ○­ Sanz et al. [30];  Zivkovi et al. [31]; Rahman et al. [32]; * Knezevic-Stevanovic et al. [33]. Dotted lines represent the expanded uncertainty of the vibrating wire viscometer.

Assael et al. [34] measured viscosities of 1-butanol at T = (298.15 and 323.15) K and p ≤25 MPa at different pressures. Also, Papaioannou et al. [35] measured viscosities of 1-butanol at T = 298.15 K and p ≤52 MPa. We compared their experimental values with those calculated using the parameters of the modified VFT model given in table 3, and obtained an average absolute relative deviation of 1.9% for [34] and 0.3% for [35]. As can be seen in figure 4, deviations obtained at 298.15 K in comparison with [34] are slightly higher than the uncertainty. On the other hand, the agreement is excellent in comparison with [35] at the same temperature.

Figure 4. Relative deviations of viscosities in comparison with the values calculated using modified VFT equation (eq. 2) for 1-butanol as a function of pressure: this work □ at T = 298.15 K and ■ at T = 323.15 K; Assael et al. [34] ○ at T = 298.15 K and ● at T = 323.15 K; Papaionoannou et al. [35] ◊ at T = 298.15 K.

As regards the behaviour of (1-butanol + hydrocarbon) mixtures, the viscosities for the binary systems {1-butanol (x) + 2,2,4-trimethylpentane (1-x) } and {1-butanol (x) + 1,2,4-trimethylbenzene (1-x) } are reported in tables 4 and 5 at x = (0.3, 0.5, 0.8) at wide ranges of pressure and temperature.

Table 4. Experimental dynamic viscosity, mPa·s), for the mixtures of {1-butanol (x) + 2,2,4-trimethylpentane (1-x) } as a function of pressure and temperaturea.

|  | mPa·s |  |  |  |
| --- | --- | --- | --- | --- |
|  | T / K |  |  |  |
| p /MPa | 293.15 K | 313.15 K | 333.15 K | 353.15 K |
|  | x = 0.3002 |  |  |  |
| 0.1 | 0.6510 | 0.4938 | 0.3855 | 0.3085 |
| 1.0 | 0.6579 | 0.4951 | 0.3922 | 0.3131 |
| 5.0 | 0.6912 | 0.5303 | 0.4188 | 0.3362 |
| 10.0 | 0.7318 | 0.5681 | 0.4439 | 0.3574 |
| 20.0 | 0.8168 | 0.6344 | 0.4973 | 0.3982 |
| 30.0 | 0.9098 | 0.7073 | 0.5520 | 0.4475 |
| 40.0 | 1.0037 | 0.7728 | 0.6073 | 0.4910 |
| 50.0 | 1.1077 | 0.8419 | 0.6677 | 0.5403 |
| 60.0 | 1.2148 | 0.9375 | 0.7297 | 0.5920 |
| 70.0 | 1.3241 | 1.0048 | 0.7944 | 0.6322 |
| 80.0 | 1.4330 | 1.0809 | 0.8570 | 0.6788 |
| 100.0 | 1.6792 | 1.2353 | 0.9892 | 0.7915 |
| 120.0 | 1.9417 | 1.4135 | 1.1334 | 0.8951 |
| 140.0 | 2.2110 | 1.5872 | 1.2794 | 1.0073 |
|  | x = 0.5003 |  |  |  |
| 0.1 | 0.8986 | 0.6427 | 0.4739 | 0.3658 |
| 1.0 | 0.9064 | 0.6458 | 0.4756 | 0.3715 |
| 5.0 | 0.9556 | 0.6742 | 0.5003 | 0.3955 |
| 10.0 | 0.9979 | 0.7086 | 0.5334 | 0.4195 |
| 20.0 | 1.1399 | 0.8024 | 0.5894 | 0.4654 |
| 30.0 | 1.2741 | 0.8924 | 0.6494 | 0.5151 |
| 40.0 | 1.4151 | 0.9864 | 0.7134 | 0.5714 |
| 50.0 | 1.5621 | 1.0844 | 0.7814 | 0.6195 |
| 60.0 | 1.7386 | 1.1864 | 0.8534 | 0.6790 |
| 70.0 | 1.8946 | 1.2924 | 0.9294 | 0.7319 |
| 80.0 | 2.0546 | 1.4024 | 1.0094 | 0.7953 |
| 100.0 | 2.3866 | 1.6344 | 1.1814 | 0.9151 |
| 120.0 | 2.7346 | 1.8824 | 1.3694 | 1.0260 |
| 140.0 | 3.0986 | 2.1464 | 1.5734 | 1.1396 |
|  | x = 0.8000 |  |  |  |
| 0.1 | 1.7768 | 1.1512 | 0.7766 | 0.5523 |
| 1.0 | 1.8049 | 1.1611 | 0.7891 | 0.5621 |
| 5.0 | 1.8374 | 1.2164 | 0.8182 | 0.5811 |
| 10.0 | 1.9489 | 1.2788 | 0.8680 | 0.6183 |
| 20.0 | 2.1606 | 1.4021 | 0.9602 | 0.6708 |
| 30.0 | 2.3535 | 1.5404 | 1.0400 | 0.7334 |
| 40.0 | 2.5778 | 1.6740 | 1.1321 | 0.7930 |
| 50.0 | 2.8262 | 1.8245 | 1.2315 | 0.8609 |
| 60.0 | 3.0379 | 1.9723 | 1.3283 | 0.9278 |
| 70.0 | 3.3084 | 2.1136 | 1.4194 | 0.9984 |
| 80.0 | 3.5368 | 2.2830 | 1.5314 | 1.0690 |
| 100.0 | 4.0663 | 2.6193 | 1.7347 | 1.2026 |
| 120.0 | 4.6535 | 2.9540 | 1.9733 | 1.3654 |
| 140.0 | 5.3221 | 3.3279 | 2.2125 | 1.5259 |

a Standard uncertainties: ur(p) = 0.0002 kPa/kPa; u(T) = 10 mK; u(x) = 0.0001; ur() = 0.0075 mPa·s/mPa·s.

Table 5. Experimental dynamic viscosity, mPa·s), for the mixtures of {1-butanol (x) + 1,2,4-trimethylbenzene (1-x)} as a function of pressure and temperaturea.

|  | mPa·s |  |  |  |
| --- | --- | --- | --- | --- |
|  | T / K |  |  |  |
| p /MPa | 293.15 K | 313.15 K | 333.15 K | 353.15 K |
|  | x = 0.2999 |  |  |  |
| 0.1 | 1.0044 | 0.7478 | 0.5848 | 0.4547 |
| 1.0 | 1.0191 | 0.7546 | 0.5927 | 0.4576 |
| 5.0 | 1.0660 | 0.7792 | 0.6096 | 0.4793 |
| 10.0 | 1.1039 | 0.8286 | 0.6356 | 0.4978 |
| 20.0 | 1.1968 | 0.8853 | 0.6805 | 0.5414 |
| 30.0 | 1.2916 | 0.9596 | 0.7326 | 0.5770 |
| 40.0 | 1.3912 | 1.0253 | 0.7951 | 0.6166 |
| 50.0 | 1.5157 | 1.1054 | 0.8476 | 0.6590 |
| 60.0 | 1.6503 | 1.1811 | 0.9060 | 0.7031 |
| 70.0 | 1.8042 | 1.2706 | 0.9707 | 0.7463 |
| 80.0 | 1.9383 | 1.3566 | 1.0297 | 0.7895 |
| 100.0 | 2.2361 | 1.5314 | 1.1766 | 0.8968 |
| 120.0 | 2.5783 | 1.7529 | 1.3082 | 0.9970 |
| 140.0 | 2.9557 | 1.9686 | 1.4472 | 1.1166 |
|  | x = 0.5289 |  |  |  |
| 0.1 | 1.2272 | 0.8685 | 0.6443 | 0.4950 |
| 1.0 | 1.2281 | 0.8775 | 0.6496 | 0.4993 |
| 5.0 | 1.2903 | 0.9012 | 0.6816 | 0.5174 |
| 10.0 | 1.3481 | 0.9526 | 0.7029 | 0.5381 |
| 20.0 | 1.4616 | 1.0290 | 0.7530 | 0.5739 |
| 30.0 | 1.5884 | 1.1160 | 0.8089 | 0.6155 |
| 40.0 | 1.7301 | 1.2010 | 0.8688 | 0.6521 |
| 50.0 | 1.8695 | 1.2930 | 0.9271 | 0.6945 |
| 60.0 | 2.0307 | 1.4022 | 0.9904 | 0.7392 |
| 70.0 | 2.1965 | 1.4925 | 1.0594 | 0.7855 |
| 80.0 | 2.4009 | 1.6115 | 1.1310 | 0.8338 |
| 100.0 | 2.7733 | 1.8208 | 1.2953 | 0.9482 |
| 120.0 | 3.2609 | 2.0651 | 1.4528 | 1.0623 |
| 140.0 | 3.7185 | 2.3673 | 1.6150 | 1.1936 |
|  | x = 0.7991 |  |  |  |
| 0.1 | 1.9586 | 1.2541 | 0.8537 | 0.6105 |
| 1.0 | 1.9768 | 1.2707 | 0.8605 | 0.6180 |
| 5.0 | 2.0572 | 1.3318 | 0.8976 | 0.6396 |
| 10.0 | 2.1770 | 1.3849 | 0.9400 | 0.6670 |
| 20.0 | 2.3813 | 1.5057 | 1.0264 | 0.7229 |
| 30.0 | 2.5630 | 1.6447 | 1.1069 | 0.7805 |
| 40.0 | 2.7573 | 1.7593 | 1.1968 | 0.8397 |
| 50.0 | 2.9951 | 1.9206 | 1.2820 | 0.9004 |
| 60.0 | 3.2411 | 2.0462 | 1.3791 | 0.9627 |
| 70.0 | 3.4810 | 2.1871 | 1.4665 | 1.0264 |
| 80.0 | 3.7543 | 2.3425 | 1.5774 | 1.0916 |
| 100.0 | 4.2890 | 2.6496 | 1.7711 | 1.2263 |
| 120.0 | 4.8493 | 2.9949 | 1.9450 | 1.3666 |
| 140.0 | 5.4698 | 3.3359 | 2.1353 | 1.5122 |

a Standard uncertainties: ur(p) = 0.0002 kPa/kPa; u(T) = 10 mK; u(x) = 0.0001; ur() = 0.0075 mPa·s/mPa·s.

The modified VFT model (eq.(2)) was also successfully applied to fit the experimental viscosities of these mixtures at each composition. The corresponding parameters and standard deviations are summarized in table 6 obtaining an average absolute relative deviation which ranges from 0.83% to 1.30%. As examples, the experimental viscosity for the equimolar mixtures (1-butanol + 2,2,4 trimethylpentane) and (1-butanol + 1,2,4 trimethylbenzene) are shown graphically in figures 5 and 6, together with the VFT fitting equation. In the case of the mixtures, there is only one paper [9] which reports viscosities for the mixture with 2,2,4 trimethylpentane at 0.1 MPa, the average absolute relative deviation being 2.5% which is slightly higher than the uncertainties.

Table 6: Correlation parameters of VFT model, equation (2), and standard deviations () for the mixtures: 1-butanol (x) + 2,2,4-trimethylpentane (1-x) or + 1,2,4-trimethylbenzene (1-x).

| 1-Butanol (x) | + 2,2,4-trimethylpentane (1-x) |  |  | + 2,2,4-trimethylbenzene (1-x) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | x = 0.3 | x = 0.5 | x = 0.8 | x = 0.3 | x = 0.5 | x = 0.8 |
| A / mPa·s | 0.011784 | 0.010460 | 0.010464 | 0.016279 | 0.013864 | 0.011366 |
| B / K | 1061.195 | 1063.730 | 1068.284 | 1062.108 | 1064.579 | 1068.336 |
| C / K | 28.733 | 54.390 | 85.225 | 35.582 | 55.724 | 85.738 |
| E0 / MPa | 0.892080 | 0.752861 | 0.701445 | 0.898696 | 0.408135 | 0.589666 |
| E1 / MPa·K-1 | 0.713657 | 0.599120 | 0.680099 | 0.683318 | 1.060617 | 0.716754 |
| E2 / MPa·K-2 | -0.001057 | -0.000567 | -0.000547 | 0.001147 | 0.003709 | -0.000327 |
| F | 1.555158 | 1.680003 | 1.679671 | 2.783975 | 5.559397 | 1.797448 |
| ()/ mPa·s | 0.013 | 0.012 | 0.034 | 0.017 | 0.016 | 0.020 |

Figure 5. Dynamic viscosities of {1-butanol (0.5) + 2,2,4 trimethylpentane (0.5)} mixture as a function of pressure at different temperatures: (◊)T = 293.15 K; (□)T = 313.15 K; () T = 333.15 K; (○) T = 353.15 K. Symbols represent experimental data and solid line represents VFT correlation.

Figure 6. Dynamic viscosities of {1-butanol (0.5) + 1,2,4 trimethylbenzene (0.5)} mixture as a function of pressure at different temperatures: (◊) T = 293.15 K; (□) T = 313.15 K; () T = 333.15 K; (○) T = 353.15 K. Symbols represent experimental data and solid line represents VFT correlation.

Before discussing the behaviour of the mixtures, it is interesting to point out the behaviour of the pure hydrocarbons reported in a previous paper [10]. Viscosity of 2,2,4-trimethylpentane is lower than 1,2,4-trimethylbenzene and quantitative differences in pressure and temperature effects are also observed for both hydrocarbons.

For 2,2,4-trimethylpentane, a viscosity decrease between 44 % to 46 % is observed when temperature rises from 293.15 K to 353.15 K at any pressure. Furthermore, viscosity increases between 232 % at  T = 313.15 K to 247 % at  T = 298.15 K, when the pressure increases from 0.1 to 140 MPa. On the other hand, a higher viscosity decrease is obtained for 1,2,4-trimethylbenzene, which ranges from 49 % to 60 % at the same temperature increase although a lower increase is observed due to the rise in pressure which is higher at lower temperatures, 135% at T = 353.15 K and 199% at T = 293.15 K.

As regards {1-butanol (x) + 2,2,4 trimethylpentane (1-x)} mixtures, their viscosities decrease in the region of 52% for x = 0.3, 60% for x = 0.5 and 69 % for x = 0.8, when temperature rises from 293.15 K to 353.15 K, at any pressure. The viscosity decrease is therefore higher as the mixture is enriched in 1-butanol. In contrast, when pressure is increased from 0.1 to 140 MPa, the highest viscosity increase is always observed at the lowest temperature and the effect decreases as the mole fraction of alcohol increases. The percentage ranges are: (227-240)% at x = 0.3, (212-242)% at x = 0.5 and (176-200)% for x = 0.8. The effect is depicted in figure 7, where viscosity of these mixtures is shown as a function of mole fraction at different pressures at T = 293.15 K.

Figure 7. Experimental data of dynamic viscosity for the mixture {1-butanol (x) + 2,2,4 trimethylpentane (1-x)} at T = 293.15 K, as a function of mole fraction at different pressures: ◊ p = 0.1 MPa,  p = 20 MPa,  p = 40 MPa, ○ p = 60 MPa,  p = 80 MPa, □ p = 100 MPa,   p = 120 MPa and * p = 140 MPa. Data for x = 0 were published in [10].

The viscosity behaviour for {1-butanol (x) + 1,2,4 trimethylbenzene (1-x)} mixtures is qualitatively similar to the former mixtures. In the measured temperature range, viscosity decreases due to the increased temperature at a given pressure, and is greater as the mixture is enriched in 1-butanol without significant differences at each isobar. These decreases are in the region of 57% for x = 0.3, 63% for x = 0.5 and 70% for x = 0.8. Figure 8 shows this behaviour graphically. In addition, the increase of viscosity due to the pressure increase from 0.1 MPa to 140 MPa is more pronounced at lower temperatures: (146-194) % at x = 0.3, (141-203) % at x = 0.5 and (148-179)% for x = 0.8.

Figure 8. Experimental data of dynamic viscosity for the mixture {1-butanol (x) + 1,2,4 trimethylbenzene (1-x)} at p = 140 MPa, as a function of mole fraction at different temperatures: ◊ T = 293.15 K, □ T = 313.15 K,  T = 333.15 K and T = 353.15 K. Data for x = 0 were published in [10].

4.	Conclusions

Viscosity measurements of 1-butanol and its binary mixtures with 2,2,4- trimethylpentane or 1,2,4-trimethylbenzene were performed over a wide range of temperatures and pressures (293.15 K to 373.15 K and up to 140 MPa), using a vibrating-wire viscometer with an expanded uncertainty (k = 2) of ±1.5%.

Data were compared with the scarce extant literature data, and an agreement within uncertainty was obtained. The modified VFT equation was successfully used to fit the experimental data as a function of temperature and pressure with an average absolute deviation which agrees with measurement uncertainties.

Acknowledgements

J. Zambrano acknowledges to Secretaría Nacional de Educación Superior, Ciencia, Tecnología e Innovación (SENESCYT), Ecuador, through a scholarship for doctoral studies. The work was funded by the Spanish Ministerio de Ciencia e Innovación (MICINN) through the Project ENE2009-14644-C02-01. We thank Prof. Trusler from Imperial College of London for providing us with the sensor.

References

Communication from the Commission - an EU Strategy for Biofuels {SEC(2006) 142}/* COM/2006/0034 final */ - Available at: <http:// http://eur-lex.europa.eu/legal-content/EN/TXT/?uri=CELEX%3A52006DC0034 >

R.E.H. Simsa, W. Mabeeb, J.N. Saddler, M. Taylor, Biosource Technology 101 (2010) 15701580.

J. Cheng, Biomass to renewable energy processes. Florida: CRC Press; 2009.

S. Szwaja, J.D. Naber, Fuel 89 (2010) 15731582.

E. Hendriks, G.M. Kontogeorgis, R. Dohrn, J.-C. de Hemptinne, I.G. Economou, L. Fele Žilnik, V. Vesovic, Ind. Eng. Chem. Res. 49 (2010): 1113111141.

A. Belabbaci, R.M. Villamañán, L. Negadi, M.C. Martín, A. Ait Kaci, M.A. Villamañán, J. Chem. Eng. Data 57 (2012) 114–119.

S. Ghellai, A. Belabbaci, R.M. Villamañán, M.C. Martín, M.A. Villamañán, L. Negadi, J. Chem. Thermodyn 63 (2013) 164–168.

F. Aguilar, F. Alaoui, J.J. Segovia, M.A. Villamañán, E.A. Montero, Fluid Phase Equilib. 315 (2012) 1–8.

G.A. Torín-Ollarves, J.J. Segovia, M.C. Martín, M.A. Villamañán, J. Chem. Thermodyn. 44 (2012) 7583.

J. Zambrano, M. Sobrino, M.C. Martín, J.J. Segovia, M.A. Villamañán, C.R. Chamorro, J. Chem. Thermodyn. 96 (2016) 104116.

J. Zambrano, “Desarrollo de un viscosímetro de hilo vibrante para la caracterización termofísica a alta presión de nuevos biocombustibles”, PhD Thesis, University of Valladolid, Spain, 2014.

M.J.Assael, C.P. Oliveira, M. Papadaki, W.A. Wakeham, Int. J. Thermophys. 13 (1992) 593–615.

F. Peleties, J.P.M. Trusler, J. Chem. Eng. Data; 56 (2011) 2236–2241.

T. Retsina, S.M. Richardson, W.A. Wakeham, Applied Sci. Res. 43 (1986) 127158.

T. Retsina, S.M. Richardson, W.A.Wakeham, Applied Sci. Res. 43 (1987) 325346.

H. Vogel, Physikalische Zeitschrift 22 (1921) 645–646.

G.S. Fulcher, J. American Ceramic Society; 8 (1925) 339355.

G. Tammann, W. Hesse, Zeitschrift für anorganische und allgemeine Chemie 156 (1926) 245–257.

C.A. Angell, W. Sichina, Annals of the New York Academy of Sciences 279 (1976) 53–67.

C.A. Angell, Pure and Applied Chemistry 63 (1991) 13871392.

G. Adam, J.H. Gibbs, J. Chem. Physics 43 (1965) 139–146.

M.J.P. Comuñas, A. Baylaucq, C. Boned, J. Fernández, Int. J. Thermophys. 22 (2001) 749–768.

X. Paredes, A.S. Pensado, M.J.P. Comuñas, J. Fernández, J. Chem. Eng. Data 55 (2010) 4088-4094.

J.A. Riddick, W.B. Bunger, T.K. Sakano, Organic solvents: physical properties and methods of purification, New York John Wiley & Sons, 1986, 4th ed.

M.A. Saleh, M. Habibullah, M. Shamsuddin Ahmed, M. Ashraf Uddin, S.M.H. Uddin, M. Afsar Uddin, F.M. Khan, Phys. Chem. Liq. 43 (2005) 485494.

N.N.Wankhede, M.K. Lande, B.R. Arbad, J. Chem. Eng. Data 50 (2005) 969972.

J.T. Chen, W.L. Weng, H.P. Chu, J. Chem. Eng. Data 51 (2006) 14411445.

A.A. Rostami, M.J. Chaichi, M. Sharifi, Monatshefte fur Chemie 138 (2007) 967971.

R. Sadeghi, S. Azizpour, J. Chem. Eng. Data 56 (2011), 240–250. DOI: 10.1021/je100818t

L.F. Sanz, J.A. González, I. García de la Fuente, J.C. Cobos, J. Mol. Liquids 172 (2012) 2633.

N. Živković, S. Šerbanović, M. Kijevčanin, E. Živković, Int. J. Thermophys. 2013, 34 (2013) 10021020.

M.S. Rahman, F.I. Chowdhury, M.S. Ahmed, M.M.H. Rocky, S. Akhtar, J. Mol. Liquids 190 (2014) 208214.

A. B. Knežević-Stevanović, I.R. Radović, S.P. Šerbanović, M. Lj. Kijevčanin, J. Chem. Eng. Data 59 (2014) 4133–4150.

M.J. Assael, S.K. Polimatidou, Int. J. Thermophys. 15 (1994) 95107.

D. Papaioannou, C. Panayioton, J. Chem. Eng. Data 40 (1995) 202-209.